BL-K1 Brushless DC Controller (DC24) User Manual

- Precautions

- The 0V (negative pole) of the driver card power supply is grounded

- Before powering on, please make sure the positive and negative poles of the driver card power supply are connected correctly.

- Do not use in explosive environments, flammable gas environments, corrosive environments, places exposed to water, or near flammable materials.

- Do not touch the connection terminals of the driver card immediately after turning off the power (within 30 seconds), as residual voltage may cause electric shock.

- When setting up, connecting, running, operating, checking, and troubleshooting, all operations must be performed by qualified personnel.

- Please do not move, set up, connect, check, etc. when the power is on. When operating, be sure to cut off the power before operating.

Environmental requirements: Indoor (no direct sunlight, rain, water droplets, condensation and corrosive gas))

Main Features

- Constant speed, start and stop, forward and reverse

- Analog speed regulation and IO control

- Acceleration/deceleration time adjustment

- Overvoltage/undervoltage/overcurrent protection

- Multiple LED alarm indicators

Usage Environment

| Operating ambient temperature | -10-40℃ |
| --- | --- |
| Operating environment humidity | 10%-90%RH (no frost) |

Electrical parameters

|  | Numerical | default value | unit |
| --- | --- | --- | --- |
| Current | < 6 | 5 | A |
| Pole pairs | --- | 4 | --- |
| Acceleration and deceleration | 10-3000 | 50 | ms |
| Applicable speed | --- | 3000 | RPM |

Dimensions

Interface Function

| Features and functions |  |  | Remark |
| --- | --- | --- | --- |
| powered by | Voltage | Normal operating voltage 24V±5% |  |
|  |  | - Start and stop control: connect 5V or disconnect to control running or stopping - Direction control: connect 5V or disconnect to control forward or reverse - Speed ​​control: When RV rotates counterclockwise ≤5%, VSP control is effective When RV rotates clockwise ≥5%, RV control is effective - Status indication: Controller abnormal ALM alarm 5V output The indicator lights flash alternately (see the attached table for details) |  |

Wiring Diagram

Function/mode diagram

Use external analog speed control DC (0~5V)

When switching to external analog speed control mode, the built-in potentiometer RV must be in the off state. This means turning the built-in potentiometer RV counterclockwise to the <5% limit. The factory default maximum speed of the drive is 3000 rpm. To change this default, please contact us.

The right picture is the curve of a certain external analog speed regulation linearity test

When switching to external analog speed control mode, the built-in potentiometer RV must be in the off state. This means turning the built-in potentiometer RV counterclockwise to the <5% limit. The factory default maximum speed of the drive is 3000 rpm. To change this default, please contact us.

The right picture is the curve of a certain external analog speed regulation linearity test

When switching to external analog speed control mode, the built-in potentiometer RV must be in the off state. This means turning the built-in potentiometer RV counterclockwise to the <5% limit. The factory default maximum speed of the drive is 3000 rpm. To change this default, please contact us.

The right picture is the curve of a certain external analog speed regulation linearity test

When switching to external analog speed control mode, the built-in potentiometer RV must be in the off state. This means turning the built-in potentiometer RV counterclockwise to the <5% limit. The factory default maximum speed of the drive is 3000 rpm. To change this default, please contact us.

The right picture is the curve of a certain external analog speed regulation linearity test

Use PWM speed control

PWM speed control input, with a duty cycle ranging from 2% to 95% for speed control. When the duty cycle is greater than 95%, the motor speed reaches its maximum value. The maximum speed value depends on the motor specifications and power supply voltage (duty cycle pulse frequency range: 10-50kHz, waveform: pulse wave, amplitude: 0-5V, note: high level 2.2V-5V, low level 0-0.7V). Note: Only supports PLCs with a signal voltage of 5V.

Status indicator and abnormal handling

| 1 green 4 red | Driver voltage is too high | The power supply voltage is too high |
| --- | --- | --- |
| 1 green 5 red | Driver overcurrent | Motor wiring error or MOS short circuit |
| 1 green 6 red | Motor open circuit | The motor power line is not connected properly |
| 2 green 3 red | Internal voltage error | Driver internal circuit failure |
| 2 green 4 red | Driver voltage is too low | The supply voltage is lower than 15VDC |
| 2 green 5 red | Motor overload | External load is too large |
| 2 green 6 red | Hall signal error | Hall signal line is not connected properly |
| 1 green 7 red | Communication Error | The communication line is not connected properly |
| 2 green 7 red | FLASH read/write errors |  |