BLK2-GT1 Electric Roller (DC 24V) Driver Card User Manual

- Precautions

- The 0V (negative pole) of the driver card power supply is grounded

- Before powering on, please make sure the positive and negative poles of the driver card power supply are connected correctly.

- Do not use in explosive environments, flammable gas environments, corrosive environments, places exposed to water, or near flammable materials.

- Do not touch the connection terminals of the driver card immediately after turning off the power (within 30 seconds), as residual voltage may cause electric shock.

- When setting up, connecting, running, operating, checking, and troubleshooting, all operations must be performed by qualified personnel.

- Please do not move, set up, connect, check, etc. when the power is on. When operating, be sure to cut off the power before operating.

Environmental requirements: Indoor (no direct sunlight, rain, water droplets, condensation and corrosive gas)

Main Features

- Constant speed, start and stop, forward and reverse

- Dial speed regulation and IO control

- Acceleration/deceleration time adjustment

- Overvoltage/undervoltage/overcurrent protection

- Multiple LED alarm indicators

Usage Environment

| Operating ambient temperature | -10-40℃ |
| --- | --- |
| Operating environment humidity | 10%-90%RH (no frost) |

Electrical parameters

| Brushless motor for electric roller |  |  |  |
| --- | --- | --- | --- |
|  | Numerical | default value | unit |
| Current | --- | 4.0 | A |
| Pole pairs | --- | 2 | --- |
| Maximum speed | 4955 | --- | RPM |

Interface Definition

Interface Functio

Interface Function

| Features and functions |  |  | Remark |
| --- | --- | --- | --- |
| powered by | Voltage | Normal operating voltage 24V±5% |  |
| Signal port | Name (English) | describe |  |
|  | PLS | Speed ​​output signal |  |
|  | ERR | Alarm output signal |  |
|  | V_IN | Analog input signal |  |
|  | DIR | Direction input signal |  |
|  | RUN | Start input signal |  |

Rotary encoder

| Knob position (Gear) | Motor speed (RPM) | Speed ​​knob | Knob position (Gear) | Motor speed (RPM) |
| --- | --- | --- | --- | --- |
| 0 | 619 |  | 5 | 3299 |
| 1 | 825 |  | 6 | 3741 |
| 2 | 1246 |  | 7 | 4121 |
| 3 | 1656 |  | 8 | 4563 |
| 4 | 2490 |  | 9 | 4955 |

Acceleration and deceleration potentiometer

| VR1-ACC VR2-DEC | Acceleration and deceleration (milliseconds) |  |
| --- | --- | --- |
| Minimum | 100 |  |
| Maximum | 300 |  |

DIP switch

| Dip switch | illustrate | ON | OFF | default |
| --- | --- | --- | --- | --- |
| SW1 | Automatic/manual reset switch | Manual reset | Automatic reset | ON |
| SW2 | Internal/external speed setting selection | external | internal | OFF |
| SW3 | Motor rotation direction | Reverse direction | Direction is not reversed | OFF |
| SW4 | Alarm signal output polarity | Output when no alarm | Output when an alarm occurs | OFF |
| SW5 | Braking method | Servo brake | Electromagnetic brake | OFF |
| SW6 | Alarm output signal type | PNP output | NPN output | ON |

Control mode settings

| NPN type connection | PNP type connection |
| --- | --- |

Power connection

LED light indication

| LED Status | describe | Cause of failure |
| --- | --- | --- |
| Green light is always on | Motor enable | ------------------ |
| Red light is on | Motor not enabled | ------------------ |
| Flashing green light | The motor is running | ------------------ |
| 1 green 1 red | Motor overspeed | The motor pole pair number does not match or stalls |
| 1 green 2 red | Commutation failure | The motor is stalled or the Hall effect is faulty during the commutation process. |
| 2 green 1 red | Motor stall | External torque is too large |
| 1 green 3 red | Drive temperature is too high | The working environment temperature is too high |
| 1 green 4 red | Driver voltage is too high | The power supply voltage is too high |
| 1 green 5 red | Driver overcurrent | Motor wiring error or MOS short circuit |
| 1 green 6 red | Motor open circuit | The motor power line is not connected properly |
| 2 green 3 red | Internal voltage error | Driver internal circuit failure |
| 2 green 4 red | Driver voltage is too low | The supply voltage is lower than 15VDC |
| 2 green 5 red | Motor overload | External load is too large |
| 2 green 6 red | Hall signal error | Hall signal line is not connected properly |
| 1 green 7 red | Communication Error | The communication line is not connected properly |
| 2 green 7 red | FLASH read/write errors | ------------------ |